BOM

| Reference | Value | Manufacturer No | Qty |
| --- | --- | --- | --- |
| C1,C9 | 22n | K223K15X7RF5UH5 | 2 |
| C2 | 1n | K102J15C0GF53L2 | 1 |
| C4 | 100p | K101J15C0GF53L2 | 1 |
| C5 | 2.2u | 50ML2R2MEFC4X5 | 1 |
| C6,C7 | 4.7u | 50MS54R7M0557 | 2 |
| C8 | 3.3n | MCRR50332X7RK0100 | 1 |
| C10,C13,C19 | 1u | MCGPR50V105M5X11 | 3 |
| C11 | 100u | 25YXJ100M5X11 | 1 |
| C12 | 0.1u | K104K15X7RF53L2 | 1 |
| C14 | 10u | MCGPR25V106M5X11 | 1 |
| C15 | 22u | MCGPR25V226M5X11 | 1 |
| C16 | 68p | K680J15C0GF53L2 | 1 |
| C17 | 47p | K470J15C0GF53L2 | 1 |
| C18, C21 | 470n | MCRR50474Z5UM0050 | 2 |
| C20 | 220n | K224K20X7RF53L2 | 1 |
| C22 | 33p | K330J15C0GF53L2 | 1 |
| D1,D2 | 1N914 | 1N914 | 2 |
| D3 | 1N4002 | 1N4002G | 1 |
| D4,D5,D6 | Red | MP006838 | 3 |
| D7,D8 | BAT41 | BAT41 | 2 |
| J1,J2 | AudioJack2 | NYS229 | 2 |
| J6 | Barrel_Jack | PPW01004 | 1 |
| Q1 | J111 | J111 | 1 |
| R1,R2,R8,R14,R18 | 1M | MF12 1M | 5 |
| R3,R6 | 1K | MF50 1K | 2 |
| R4,R10 | 47Ω | MF50 47R | 2 |
| R5 | 560Ω | MF50 560R | 1 |
| R7 | 1.5K | MCF 0.5W 1K5 | 1 |
| R9 | 10K | MF50 10K | 1 |
| R11,R12 | 100K | MF50 100K | 2 |
| R13 | 2.2k | MF50 2K2 | 1 |
| R15,R16,R17 | 22k | MF50 22K | 3 |
| RV1-GAIN1,RV2-FILTER1 | B100K | RD901F-40-15K-B100K | 2 |
| RV3-VOL1 | A100K | RD901F-40-15K-A100K | 1 |
| RV4-BLEND1 | B100K | RD902F-40-15K-B100K | 1 |
| S1 | SWITCH-3PDT |  | 1 |
| SW3 | SW_Rotary4x3 | CK1052 | 1 |
| SW8 | SW_Rotary2x6 | CK1050 | 1 |
| U1 | OP07CP | OP07CP | 1 |
| U2,U3 | TL072ACP | TL072CP | 2 |
| Yellow Enclosure | 1590 | 1590BBYL | 1 |

Soldering guide

Break off the two sub-boards (2: tone selecta, 3: stompy) from the main board (1: the meat and potatoes). The break-points are labelled on the bottom side of the board.

With reference to the BOM, work through soldering each component. Start with low-profile components (resistors, diodes, capacitors) and work up to high-profile components (potentiometers, rotary switches).

Leave the connections between boards, and with the off-board components (audio and power jacks), they will be added last.

FIXES FOR VERSION 1.2:

- Connection from C1(1) and J3(4), and between C19(2) and J3(5), signal in and out of circuit.

- Test points more obvious?

- Rotary switch footprints (A,B,C,D, get placement correct, line up pin 1)

- Move most components to underside of board, should make for easier troubleshooting and less cramped.

- Better order of diode selection (not switching both diodes at once)

- Potentiometer directionality, Gain, Filter

- Wet/Dry functionality (buffer was in wrong configuration, input into -) Also check biasing of this section…

- Silkscreens, some components were missing

- Soft/hard clipping switch

- Replace 2x6 diode selection switch with 1x12 as the two poles are connected anyway and this could allow 12 different diode type pairings

- Active treble cut/boost instead of passive filter, OR implement tilt EQ (intense)

NOTE:

Any boards marked V1.1 have a grave error, the input and output pins for the entire effects circuit are disconnected.

This can be fixed by throwing a wire between C1(1) and J3(4), and between C19(2) and J3(5).

Enclosure guide

Top side

Bottom Side